1: yx+xy+yx

2: YX+YX→Absorbtion Law (A+AB=A)

3: Y(X+X)→Distributive Law (AB+AC=A(B+C)

4: Y1→Complement Law (A+A=1)

5: Y→Identity Law (A1=1)

6: Solution = y

1: AB*B+B*AB

2: ABB+B*AB→Involution law

3: ABB+BA+B→Demorgan Theorm

4: 0+BA+B→Complement law AA=0

5: BA+BB

6: BA+0

7: BA

Function extracted: ABC+ABC+ABC+ABC+ABC

I can be reduced to: B+AC

Extracted expression: ABC+ABC+ABC

Simplification process:

1: ABC+ABC+ABC

2: ABC+BCA+A→Distributive Law (AB+AC=A(B+C)

3: ABC+BC1→Complement law AA=0

4: ABC+BC→Identity Law (A1=1)

Logisim solution:

| A | B | CARRY | SUM |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 |

Simulation with chips:

Implementation on Breadboard:

1st Setup:  Result, no lights – as expected
Input:
- A is Negative (Green wires)
- B is Negative (Purple wires)

2nd Setup:  Result, Sum light shines – as expected
Input:
- A is Positive (Green wires)
- B is Negative (Purple wires)

3rd Setup:  Result, Sum light shines – as expected
Input:
- A is Negative (Green wires)
- B is Positive (Purple wires)

4th Setup:  Result, CarryOver light shines – as expected
Input:
- A is Positive (Green wires)
- B is Positive (Purple wires)

- Full-adder gate

  - Theory

(5)

(4)

(3)

(2)

(1)

Draw the truth-table describing the logic from a full adder.

Truth-table:

| A | B | Cin | A⨁B (1) | AB (2) | (A⨁B)⨁Cin (3) | (A⨁B)Cin (4) | A⨁BCin+AB (5) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A exclusive OR B | A and B | Sum | - | Cout |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |

Exclusive or: False when both inputs are the same, True when both inputs are different.

  - Simulation

Do it in Logisim, using both ideal and ’real’ components.

Ideal components:

Real components:

  - Realization

To realize the full adder an OR gate is also needwed. This can be done with SN74HCT32N.

- Create a circuit on the breadboard.

- Test that it works as described in the truth table in section 3.1

NOTE: UNABLE TO COMPLETE DUE TO LACK OF CABLES!

- 2’s Complement Exercise

- All positive numbers begin with a 0

- All negative numbers begin with a 1

- To convert x to –x: flip all bits of x, add 1 and throw away the carry (if any)

- To calculate Y – X (both X and Y >= 0):

  - Calculate 2’s complement of X

  - Add that to Y

  - 8-bit processor

The following binary pattern is stored in the RAM: 0b11100110

In case this should be interpreted as an unsigned integer. What is the value?

Since unsigned integers can only be positive, we can simply convert the binary pattern to its corresponding decimal value:

Convert to decimal:

Decimal=1* 27+1* 26+1* 25+0* 24+0* 23+1* 22+1* 21+0* 20

Decimal=1* 27+1* 26+1* 25+1* 22+1* 21

Decimal=128+64+32+4+2=230

In case this should be interpreted as a signed integer. What is the value?

- Find 2’s Complement by inverting all the bits:
0b11100110 -> 0b00011001 (it is now a positive number)

- Add 1 to the inverted bits, and throw away any carry:

00011001 + 00000001 -> 00011010

Convert to decimal:

Decimal=0* 27+0* 26+0* 25+1* 24+1* 23+0* 22+1* 21+0* 20

Decimal=1* 24+1* 23+1* 21

Decimal=16+8+2=26

- Invert the decimal value, because the initial binary pattern was negative.

Solution = -26

  - The following numbers are saved as a 8-bit signed integer type. What is the saved binary pattern?
Note: Remember, negative signed integers begin with 1. Positives begin with 0.

- -4

- Binary Conversion Algorithm is utilized on the non-negative value 4:

| 410 | 410/2 | 410 mod 2 |
| --- | --- | --- |
| 4 | 2 | 0 |
| 1 | 1 | 0 |
| 0 | 0 | 1 |
| Binary value is: 100 -> 0b00000100 |  |  |

- Since initial value was negative, we invert all the bits:
0b00000100 -> 0b11111011

- Then we add 1 to the binary:

0b11111011 + 0b00000001 = 0b11111100

- -4 is represented by the signed binary 0b11111100

- 54

- Binary Conversion Algorithm is utilized on the non-negative value 54:

| 5410 | 5410/2 | 5410 mod 2 |
| --- | --- | --- |
| 54 | 27 | 0 |
| 27 | 13 | 1 |
| 13 | 6 | 1 |
| 6 | 3 | 0 |
| 3 | 1 | 1 |
| 1 | 0 | 1 |
| Binary value is: 110110 -> 0b00110110 |  |  |

No need for further actions, since the number is positive.

- 54 is represented by the signed binary 0b00110110

- -54

- Binary Conversion Algorithm is utilized on the non-negative value 54:

| 5410 | 5410/2 | 5410 mod 2 |
| --- | --- | --- |
| 54 | 27 | 0 |
| 27 | 13 | 1 |
| 13 | 6 | 1 |
| 6 | 3 | 0 |
| 3 | 1 | 1 |
| 1 | 0 | 1 |
| Binary value is: 110110 -> 0b00110110 |  |  |

- Since initial value was negative, we invert all the bits:
0b00110110 -> 0b11001001

- Then we add 1 to the binary:

0b11001001 + 0b00000001 = 0b11001010

- -54 is represented by the signed binary 0b11001010

- -128
Note: 8-bit binary can express values from -128 to 127!

- Binary Conversion Algorithm is utilized on the non-negative value 128:

| 12810 | 12810/2 | 12810 mod 2 |
| --- | --- | --- |
| 128 | 64 | 0 |
| 64 | 32 | 0 |
| 32 | 16 | 0 |
| 16 | 8 | 0 |
| 8 | 4 | 0 |
| 4 | 2 | 0 |
| 2 | 1 | 0 |
| 1 | 0 | 1 |
| Binary value is: 10000000 -> 0b10000000 |  |  |

- Since initial value was negative, we invert all the bits:
0b10000000 -> 0b01111111

- Then we add 1 to the binary:

0b01111111+ 0b00000001 = 0b10000000

- -54 is represented by the signed binary 0b10000000

- 127

- Binary Conversion Algorithm is utilized on the non-negative value 127:

| 12710 | 12710/2 | 12710 mod 2 |
| --- | --- | --- |
| 127 | 63 | 1 |
| 63 | 31 | 1 |
| 31 | 15 | 1 |
| 15 | 7 | 1 |
| 7 | 3 | 1 |
| 3 | 1 | 1 |
| 1 | 0 | 1 |
| Binary value is: 1111111 -> 0b01111111 |  |  |

No need for further actions, since the number is positive.

- 54 is represented by the signed binary 0b01111111

  - Adding

Add the following numbers together after converting them to binary:

- 4+7

Binary Conversion Algorithm is utilized on the non-negative value 4:

| 410 | 410/2 | 410 mod 2 |
| --- | --- | --- |
| 4 | 2 | 0 |
| 2 | 1 | 0 |
| 1 | 0 | 1 |
| Binary value is: 100 -> 0b00000100 |  |  |

Binary Conversion Algorithm is utilized on the non-negative value 7:

| 710 | 7/2 | 7 mod 2 |
| --- | --- | --- |
| 7 | 3 | 1 |
| 3 | 1 | 1 |
| 1 | 0 | 1 |
| Binary value is: 111 -> 0b00000111 |  |  |

Add the numbers together:

0b00000100 + 0b00000111 = 0b00001011

- 7-4

Binary Conversion Algorithm is utilized on the non-negative value 7:

| 710 | 7/2 | 710 mod 2 |
| --- | --- | --- |
| 7 | 3 | 1 |
| 3 | 1 | 1 |
| 1 | 0 | 1 |
| Binary value is: 111 -> 0b00000111 |  |  |

Binary Conversion Algorithm is utilized on the non-negative value 4:

| 410 | 410/2 | 410 mod 2 |
| --- | --- | --- |
| 4 | 2 | 0 |
| 2 | 1 | 0 |
| 1 | 0 | 1 |
| Binary value is: 100 -> 0b00000100 |  |  |

Add the numbers together:

0b00000111- 0b00000100= 0b00000011

- 4-7

Here we need to calculate two’s complement of -7, in order to add that to 4.

Binary Conversion Algorithm is utilized on the non-negative value 7:

| 710 | 7/2 | 710 mod 2 |
| --- | --- | --- |
| 7 | 3 | 1 |
| 3 | 1 | 1 |
| 1 | 0 | 1 |
| Binary value is: 111 -> 0b00000111 |  |  |

Binary Conversion Algorithm is utilized on the non-negative value 4:

| 410 | 410/2 | 410 mod 2 |
| --- | --- | --- |
| 4 | 2 | 0 |
| 2 | 1 | 0 |
| 1 | 0 | 1 |
| Binary value is: 100 -> 0b00000100 |  |  |

Invert the bits in 7 and add 1, so we get two’s complement:

0b00000111-> 0b11111000

0b11111000+ 0b00000001 = 0b11111001

Now add these two values:

00000100 (4)

+ 11111001 (-7)

-------

11111101 (-3)

Solution is: 11111101

- -4-7

Here we need to calculate two’s complement of -7 and -4, in order to add that to -4.

Binary Conversion Algorithm is utilized on the non-negative value 7:

| 710 | 7/2 | 710 mod 2 |
| --- | --- | --- |
| 7 | 3 | 1 |
| 3 | 1 | 1 |
| 1 | 0 | 1 |
| Binary value is: 111 -> 0b00000111 |  |  |

Binary Conversion Algorithm is utilized on the non-negative value 4:

| 410 | 410/2 | 410 mod 2 |
| --- | --- | --- |
| 4 | 2 | 0 |
| 2 | 1 | 0 |
| 1 | 0 | 1 |
| Binary value is: 100 -> 0b00000100 |  |  |

Invert the bits in 7 and add 1, so we get two’s complement:

0b00000111-> 0b11111000

-7 = 0b11111000+ 0b00000001 = 0b11111001

Invert the bits in 4 and add 1, so we get two’s complement:

0b00000100-> 0b11111011

-4 = 0b11111011+ 0b00000001 = 0b11111100

Now add these two values (throw any carryover away):

11111100 (-4)

+ 11111001 (-7)

-------

11110101 (-1)

Solution is: 11110101